2

2

16

3

4

7

9

11

5

5

18

6

17

1

1

1

1

1

2

20

17

24

Line A

1

Lx4

Lx3

Lx2

19

13

22

13

23

Lx1

14

14

Edge of stage

8

10

12

15

FOH

15

21

21

LIGHTS

- A minimum of  6 x 1k or 8 x 500  floods on backdrop (made of hessian, about 9 m wide), lee gel 119 or 363

2-   	3 x 1k  fresnels, back-light, gel 119

3-	Profile, narrow, o/w

4-	Profile, narrow, o/w with iris

5-	1k fresnell, back-light, gel 079

6-	1k fresnell, back-light, gel 161

7-	1 k fresnell, back-light, gel  020

8-	1K fresnell (or frosted profile), gel 161

- 1 k fresnell, back-light, gel lee 020

- 1K fresnell (or frosted profile), gel 161

- 1 k fresnell, back-light, gel lee 020

12- 	1K fresnell (or frosted profile),  gel 161

13- 	side lights on booms (light about 2 mt high) , gel 161

- top-side lights, wide profiles, gel 079

- 1K fresnell, gel 201 + 1 K fresnell gel  lee 205 (double )

- Profile, gel 161

- 1k fresnell, gel 132

- Profile, narrow gel 716 + profile (SOURCE 4, narrow angle like a 10 degrees is better) gel gold or dark amber (lee gel 021) with iris. We will use two (in amber with iris) if necessary.

- profile, gel 161

- pin spot or small profile with gel pale lavander

- top-side lights on first 5-10 rows of seats, gel 161

- 8 spot lights (provided by LYNGO)

- Pin spot or small profile with gel pale lavander

PRACTICALS (provided by LYNGO)

24- 	moon that lights up

25- 	light in oven

- fan

WE USE  A SMOKE MACHINE powered by a remote control

SIX CANDLES ARE LIT ON STAGE WITH TWO DISPOSABLE LIGHTERS

A SPARKLER IS LIT INSIDE A METAL SAUCEPAN

Stage: ideally about 9 m X 9 m

VERY IMPORTANT the front of house bar, our FOH, should be about one meter from the front edge of stage so when lighting  the actors downstage we do not  hit the area  beyond lineA (see plan) which is about 2 m from backdrop.

LX1 should be  close to the front edge of the stage.

A manual use of the lighting desk is what we prefer. The lights will be recorded in submasters in the same  groups and order of this lighting plan.

Sound: minidisc player with autopause.